HUANGSHAN HUIYUANMAO GREENFOOD DEVELOPMENT CO., LTD

Adress: Room 1-402, Changyuan Village, North Qianyuan Road,Tunxi District,Huangshan City, Anhui Province, China

Tel: 0086-559-2568466/2564066  Fax: 0086-559-2564066

E-mail:hscjf@vip.sina.com

Osmanthus Absoulte Oil

MATERIAL SAFETY DATA SHEET

- PRODUCT&COMPANY IDENTIFICATION

- Trade name                 Osmanthus Absolute Oil

- Use                       Used mainly in daily chemical,tabacco,liquor and candy

Manufacturer               Huangshan HuiYuanMao GreenFood Development Co.,Ltd.

Adress                    Room 1-402, Changyuan Village, North Qianyuan Road,

Tunxi District,Huangshan City, Anhui Province, China

Telephone Number          0086-559-2568466/2564066

Emergency Telephone Number   0086-13805599138

- COMPOSITION/INFORMATION INGREDIENTS

- CAS number                  977103-81-3

- HAZARDS IDENTIFICATION

- General              There is mild non-irritation when contact with skin, but contact with

- eyes may cause irration

- Hazard symbol        Not regulated

- Risk phrase           Not regulated

- S-phrase             Not regulated

- FIRST AID MEASURES

- Inhalation                    If excess inhale,remove individual to fresh air. If neccessary contact physician.

- Eye exposure                 Flush immediately with cold clean water for at least 15 minutes, contact physican immediately.

- Skin exposure               Thoroughly wash the affected area with soap and water.flush with large quantities of water. If irratation persists call a physician.

- Ingestion                  Material at the concentration supplied is not for ingestion. In case

- of ingestion see a physician. Don’t try to vomit. Rinse mouth.

- Others                    Take risk and safety phrases (section 15) into account.

- FIRE-FIGHTING MEASURES

- Extinguishing media           CO2, arbondioxide, dry chemical, foam. Don’t use direct water jet on burning material.

- ACCIDENTAL RELEASE MEASURES

- Personal precaution            Avoid inhalation and contact with skin and eyes.

- Spillage                   Small spills: wide up with paper rags and place contaminated items in closed metal waste container. Large spills: Use non flammable absorbent.

- Environmental precaution       Keep away from drains, surface and ground water and soil.

- HANDING AND STORAGE

- Handling                    Apply according to good manufacturing and industrial hygiene practices with proper ventilation. Don’t drink or excess smoke while handling. Respect good personal hygiene.

- Storage                    Store in a cool ,dry and ventilated area away from heat sources

- and protected from light in tightly closed container.

- Fire protection                Keep away from ignition sources and naked flame.

- EXPOSURE CONTROLS AND PERSONAL PROTECTION

- Respiratory protection      Avoid excessive inhalation. Apply local ventilation where apporpriate.

- Eye protection                Use goggles.

- Skin protection                Avoid skin contact.use chemical resistant gloves.

- PHYSICAL AND CHEMICAL PROPERTIES

- Appearance   Dark brown Viscious liquid   Explosive property          Not determined

- Odor natural  Natural Osmanthus fragrans   Oxidizing property          Not determined

- Color        Dark Brown               Vapour pressure             Not determined

- Ph value      Not determined            Density (25℃)             0.9100-0.9650

- Viscosity     Not determined           Solubility in water            Insoluble

- Freezing point Not determined           Solubility in ethanol          Soluble

- Boiling point  Not determined           Partition coefficient          Not determined

- Melting point  ≤15℃   Flash point of the volatile fraction ≧78℃                 Refractive index(25℃) 1.4800-1.4910   Auto inflammability            ≧280℃                Ester value        ≧40.0  mg/g        Acid value               30-100mg/g

- STABILITY AND REACTIVITY

Reactivity           Presents no significant reactivity hazard by itself or in contact with water.

Decomposition             Don’t throw in sewer or natural environment. Use local regulation.

- TOXOLXGICAL INFORMATION

Based on the available toxicological data

- ECOLOGICAL INFORMATION

Prevent contamination of soil,ground and surface water

- DISPOSAL CONDITONS

Dispose of according to local regulations.

Avoid disposal into drainage systems and into environment.

- TRANSPORT INFORMATION

UN Number               Not Regulated

Packing Group             Not Regulated

Packing Class              Not Regulated

- REGULATORY INFORMATION

Hazard Symbol             Not Regulated

Risk Phrases               Irritating to eyes

SafetyPhrases              Avoid contact with eyes

Contains                  None be reported

- OTHER INFORMATION

The information in this material safety data sheet is based on the properties of the material known to our company at the time the National Standard of PRC was issued .The GB6780-2008 is intended to provide information for a health and safety assessment of the material and circumstances under which it is packaged,stored or applied in the work place .For such a safety assessment our company hold no responsibility. This document is not intendede for quality assurance purposes.